RV32I RISC-V Core

Instruction Fetch Stage

rv32i_fetch.v  —  RTL Design Documentation

Pipeline Stage: Stage 1 of 5  |  Language: Verilog RTL  |  ISA: RV32I

Project: Angelo Jacobo (Open-Source Reference)  |  Study & Learning Documentation

# 1.  Module Overview

rv32i_fetch is the first stage (IF) of the classic 5-stage RISC-V pipeline. Its sole responsibility is to present one valid, correctly-addressed instruction to the downstream Decode stage on every cycle it is able to. Everything else — stalls, flushes, PC redirection — exists to protect that goal.

The module sits at the boundary between the pipeline control domain and the instruction memory interface. It drives a Wishbone-style strobe/acknowledge handshake (o_stb_inst / i_ack_inst) to the instruction memory and manages a single pipeline-register output (o_ce + o_inst + o_pc).

## 1.1  Role in the 5-Stage Pipeline

| Stage | Module | Key Input from IF | Key Output to IF |
| --- | --- | --- | --- |
| IF (Stage 1) | rv32i_fetch.v | — | o_inst, o_pc, o_ce |
| ID (Stage 2) | rv32i_decoder.v | o_inst, o_ce | i_stall |
| EX (Stage 3) | rv32i_alu.v | — | i_alu_change_pc, i_alu_next_pc |
| MEM (Stage 4) | rv32i_memoryaccess.v | — | i_stall (propagated) |
| WB (Stage 5) | rv32i_writeback.v | — | i_writeback_change_pc, i_writeback_next_pc |

## 1.2  High-Level Feature Summary

Five core capabilities are implemented in this module:

- PC Management — Maintains and advances the Program Counter; handles reset, sequential increment, and redirects from ALU (branch/jump) and Writeback (trap/MRET).

- Instruction Fetch Handshake — Issues o_stb_inst to instruction memory; waits for i_ack_inst; supports multi-cycle memory (no fixed-latency assumption).

- Pipeline Bubble Generation — On any PC change, forces o_ce = 0 for one or more cycles to flush wrongly-fetched instructions from entering Decode.

- Stall Handling — Freezes its own outputs when downstream stages assert i_stall. Saves the in-flight instruction and PC before the stall and restores them afterwards.

- Flush — An explicit i_flush input drives o_ce low immediately to invalidate any instruction currently at the IF/ID boundary.

# 2.  Port List & Signal Descriptions

| Port Name | Dir | Width | Group | Description |
| --- | --- | --- | --- | --- |
| i_clk | in | 1 | System | Global clock. All sequential logic is posedge-triggered. |
| i_rst_n | in | 1 | System | Active-low asynchronous reset. Clears all pipeline registers. |
| o_iaddr | out | 32 | Mem I/F | Byte address sent to instruction memory. Always word-aligned. |
| o_pc | out | 32 | Pipeline | PC value associated with the instruction in o_inst. One-cycle delayed from o_iaddr to align with the data path. |
| i_inst | in | 32 | Mem I/F | 32-bit instruction word returned by instruction memory when i_ack_inst is high. |
| o_inst | out | 32 | Pipeline | Registered instruction forwarded to the Decode stage. |
| o_stb_inst | out | 1 | Mem I/F | Wishbone-style strobe. High when the fetch stage wants a new instruction. Driven directly by internal CE register. |
| i_ack_inst | in | 1 | Mem I/F | Acknowledgment from instruction memory. Instruction on i_inst is valid when high. |
| i_writeback_change_pc | in | 1 | PC Ctrl | Asserted by WB stage to redirect PC (trap entry, MRET, or other privileged PC change). |
| i_writeback_next_pc | in | 32 | PC Ctrl | Target PC value from WB stage (trap handler address or MEPC). |
| i_alu_change_pc | in | 1 | PC Ctrl | Asserted by EX stage when a taken branch or JAL/JALR resolves. |
| i_alu_next_pc | in | 32 | PC Ctrl | Target PC value from EX stage (branch target or JALR result). |
| o_ce | out | 1 | Pipeline | Clock-enable output to Decode stage. When low, ID stage must treat its inputs as invalid (pipeline bubble). |
| i_stall | in | 1 | Pipeline | Global stall from downstream stages. When high, IF must freeze all outputs and not advance the PC. |
| i_flush | in | 1 | Pipeline | When high and not stalled, forces o_ce low at the next clock edge, discarding the current instruction. |

# 3.  Internal Signals & Registers

| Signal | Type | Description |
| --- | --- | --- |
| iaddr_d | wire/reg (comb.) | Next value of o_iaddr, computed in the combinational always block. Either PC+4, i_alu_next_pc, or i_writeback_next_pc. |
| ce | reg (seq.) | Internal fetch-enable register. High when the fetch stage is active. Drives o_stb_inst directly. |
| ce_d | wire (comb.) | Combinational next-state of ce. Cleared during PC-change cycles to create a bubble; otherwise follows ce. |
| stall_bit | wire (comb.) | OR of all stall conditions. When high, the pipeline register update is blocked. See Section 4.4 for full decode. |
| stall_fetch | reg (comb.) | Per-module stall flag. Set when a wait-state is required for memory or PC changes. Combined into stall_bit. |
| stall_q | reg (seq.) | Registered copy of the previous stall state. Used to detect the rising edge of a stall (first cycle of stall). |
| prev_pc | reg (seq.) | Delayed copy of o_iaddr. Provides one-cycle alignment so o_pc corresponds to the instruction on i_inst. |
| stalled_pc | reg (seq.) | Snapshot of prev_pc captured at the first cycle of a stall. Restored to o_pc when the stall ends. |
| stalled_inst | reg (seq.) | Snapshot of i_inst captured at the first cycle of a stall. Restored to o_inst when the stall ends. |

# 4.  Detailed Functional Description

## 4.1  PC Management

The program counter lives in o_iaddr and is updated every cycle that stall_bit is de-asserted. The combinational block computes iaddr_d (the next PC), and the sequential block latches it into o_iaddr.

Priority of PC sources (highest first):

- Writeback redirect (i_writeback_change_pc) — Traps and MRET. Writeback has architectural authority over all other stages; it wins even if ALU also requests a change.

- ALU redirect (i_alu_change_pc) — Taken branches (BEQ, BNE, BLT, BGE, BLTU, BGEU) and unconditional jumps (JAL, JALR). Resolved in the EX stage.

- Sequential (PC + 4) — Default when no redirect is active.

Note that prev_pc tracks o_iaddr with a one-cycle delay. This compensates for the memory round-trip: when the address has been issued, the data comes back one cycle later, so o_pc must report the address of that returned instruction, not the next fetch address.

## 4.2  Memory Handshake (Wishbone-like)

o_stb_inst is driven directly from ce — the fetch stage requests a new instruction whenever it is active. The memory responds by asserting i_ack_inst. Until acknowledgment arrives, stall_bit stays high and the pipeline register is frozen.

| o_stb_inst | i_ack_inst | stall_bit contribution | Effect |
| --- | --- | --- | --- |
| 0 | — | High (no request) | No instruction needed this cycle; pipeline is already stalled/bubbled. |
| 1 | 0 | High (pending) | Waiting for memory; pipeline holds current outputs. |
| 1 | 1 | Low | Instruction valid on i_inst; pipeline advances normally. |

## 4.3  Pipeline Bubble Generation

Whenever the PC must change (either source), the combinational block sets ce_d = 0. On the next clock edge, ce becomes 0, which means o_stb_inst = 0. The sequential block then propagates o_ce = 0 to the Decode stage — this is the bubble. The bubble persists for exactly as many cycles as it takes to fetch the new instruction and receive i_ack_inst.

Important: The bubble is only inserted when the stage is not already stalled (!(i_stall || stall_fetch)). If a stall is active at the same time as a PC change, the redirect is queued implicitly because iaddr_d still receives the new target and it will be latched as soon as the stall clears.

## 4.4  Stall Decode — stall_bit

The central stall wire is the OR of four independent conditions:

wire stall_bit = stall_fetch           // module-level wait

|| i_stall              // downstream stall

|| (o_stb_inst && !i_ack_inst)  // memory not ready

|| !o_stb_inst;         // not even requesting

| Condition | Meaning | Action when true |
| --- | --- | --- |
| stall_fetch | Module decided it must wait (e.g., during PC change) | Hold all pipeline registers; do not advance PC. |
| i_stall | Downstream stage (ID/EX/MEM/WB) cannot accept new data | Freeze outputs; save current instruction in stalled_inst. |
| o_stb_inst && !i_ack_inst | Request issued but memory has not replied yet | Wait; same effect as above. |
| !o_stb_inst | Fetch stage is disabled (ce = 0 / pipeline bubble) | Nothing to deliver; hold stage. |

## 4.5  Stall Save & Restore (stall_q, stalled_pc, stalled_inst)

When a stall begins (stall_bit rises for the first time, detected by stall_bit && !stall_q), the module captures a snapshot:

- stalled_pc   ← prev_pc  (the PC of the instruction currently on i_inst)

- stalled_inst ← i_inst   (the raw instruction word from memory)

On the cycle after the stall ends (stall_q goes low), o_pc and o_inst are driven from these saved registers rather than from the live prev_pc and i_inst. This is necessary because i_inst may have changed (memory bus moved on) even though the pipeline was frozen.

## 4.6  Flush

When i_flush is asserted and the pipeline is not stalled (!stall_bit), o_ce is forced to 0 on the next clock edge regardless of ce_d. The flush takes precedence over ce_d but yields to a stall (flushing a stalled stage makes no sense — the stalled instruction has not yet committed).

## 4.7  ce Register — Sequential Always Block

A separate always block manages the internal ce register:

always @(posedge i_clk, negedge i_rst_n) begin

if (!i_rst_n)

ce <= 0;

else if ((i_alu_change_pc || i_writeback_change_pc)

&& !(i_stall || stall_fetch))

ce <= 0;   // create bubble when PC changes (and not stalled)

else

ce <= 1;   // fetch stage is active in all other cases

end

Notice ce never stays 0 beyond one cycle naturally — it returns to 1 on the very next clock unless another PC-change is still active. The memory handshake (stall_bit) keeps the pipeline frozen while the new instruction is being fetched.

# 5.  Timing & Pipeline Waveform Diagrams

## 5.1  Normal Sequential Fetch (Single-Cycle Memory)

Below is the cycle-by-cycle behaviour when instruction memory always acknowledges in one cycle and no stalls or PC changes occur.

| Cycle | o_iaddr | o_stb_inst | i_ack_inst | i_inst (from mem) | o_pc | o_inst | o_ce |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N+0 (reset done) | 0x00 | 1 | 1 | INSTR_A | — | — | 0→1 |
| N+1 | 0x04 | 1 | 1 | INSTR_B | 0x00 | INSTR_A | 1 |
| N+2 | 0x08 | 1 | 1 | INSTR_C | 0x04 | INSTR_B | 1 |
| N+3 | 0x0C | 1 | 1 | INSTR_D | 0x08 | INSTR_C | 1 |

## 5.2  Branch Taken — PC Redirect from ALU

At cycle T, the EX stage resolves a taken branch and asserts i_alu_change_pc. The fetch stage must:

- Set ce = 0 (bubble) so the two wrongly-fetched instructions never reach Decode.

- Load iaddr_d with i_alu_next_pc so that the target is fetched on the next cycle.

| Cycle | i_alu_change_pc | i_alu_next_pc | o_iaddr (next) | ce / o_stb | o_ce (to ID) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| T−1 | 0 | — | PC+4 | 1 | 1 | Normal fetch |
| T | 1 | TARGET | TARGET | 0 | 0 | Bubble inserted; target loaded |
| T+1 | 0 | — | TARG+4 | 1 | 0 | Memory fetch of TARGET in flight; ce back to 1 |
| T+2 | 0 | — | TARG+8 | 1 | 1 | TARGET instruction valid at ID |

## 5.3  Load-Use Stall (i_stall from downstream)

When ID detects a load-use hazard it asserts i_stall. The fetch stage freezes, saves the in-flight instruction, and waits.

| Cycle | i_stall | stall_bit | stall_q | Action | o_inst | o_pc |
| --- | --- | --- | --- | --- | --- | --- |
| K | 0 | 0 | 0 | Normal flow | INSTR_P | PC_P |
| K+1 | 1 | 1 | 0 | First stall cycle — save stalled_inst | INSTR_Q | PC_Q |
| K+2 | 1 | 1 | 1 | Stall continues — outputs frozen | INSTR_Q | PC_Q |
| K+3 | 0 | 0 | 1 | Stall ends — restore from stalled regs | INSTR_Q | PC_Q |
| K+4 | 0 | 0 | 0 | Normal flow resumes | INSTR_R | PC_R |

# 6.  Annotated RTL Code

The complete rv32i_fetch.v source is reproduced below with inline commentary. Each logical block is labelled to correspond to the sections above.

// ═══════════════════════════════════════════════════════════════════════

//  rv32i_fetch.v  —  Instruction Fetch Stage  (Stage 1 / 5)

//  RV32I 5-Stage Pipeline  |  Angelo Jacoana Open-Source Reference

// ═══════════════════════════════════════════════════════════════════════

`timescale 1ns / 1ps

`default_nettype none

`include "rv32i_header.vh"

module rv32i_fetch #(parameter PC_RESET = 32'h00_00_00_00) (

// ── System ──────────────────────────────────────────────────────────

input  wire        i_clk, i_rst_n,

// ── Instruction Memory Interface (Wishbone-like) ─────────────────────

output reg  [31:0] o_iaddr,       // fetch address → instruction memory

output reg  [31:0] o_pc,          // PC aligned to instruction in o_inst

input  wire [31:0] i_inst,        // instruction word from memory

output reg  [31:0] o_inst,        // instruction forwarded to Decode

output wire        o_stb_inst,    // request strobe to memory

input  wire        i_ack_inst,    // acknowledge from memory

// ── PC Control Inputs ────────────────────────────────────────────────

input  wire        i_writeback_change_pc,  // redirect: trap / MRET

input  wire [31:0] i_writeback_next_pc,

input  wire        i_alu_change_pc,         // redirect: branch / jump

input  wire [31:0] i_alu_next_pc,

// ── Pipeline Control ─────────────────────────────────────────────────

output reg         o_ce,          // clock-enable to Decode stage

input  wire        i_stall,       // downstream stall

input  wire        i_flush        // flush current instruction

);

// ── Internal Registers & Wires ───────────────────────────────────────

reg  [31:0] iaddr_d;          // combinational next-PC

reg  [31:0] prev_pc;          // one-cycle delayed o_iaddr (PC alignment)

reg  [31:0] stalled_inst;     // snapshot of i_inst at stall entry

reg  [31:0] stalled_pc;       // snapshot of prev_pc at stall entry

reg         ce, ce_d;         // fetch CE: registered and combinational

reg         stall_fetch;      // module-level stall flag (combinational)

reg         stall_q;          // previous-cycle stall state

// ── Section 4.4 : Stall Logic ────────────────────────────────────────

wire stall_bit = stall_fetch

|| i_stall

|| (o_stb_inst && !i_ack_inst)

|| !o_stb_inst;

// o_stb_inst = ce : request memory whenever the stage is active

assign o_stb_inst = ce;

// ── Section 4.7 : CE Register ────────────────────────────────────────

always @(posedge i_clk, negedge i_rst_n) begin

if (!i_rst_n)

ce <= 0;

else if ((i_alu_change_pc || i_writeback_change_pc)

&& !(i_stall || stall_fetch))

ce <= 0;   // bubble: PC is changing, invalidate next fetch

else

ce <= 1;   // fetch stage is enabled

end

// ── Sections 4.1 / 4.3 / 4.5 / 4.6 : Main Sequential Block ─────────

always @(posedge i_clk, negedge i_rst_n) begin

if (!i_rst_n) begin

o_ce         <= 0;

o_iaddr      <= PC_RESET;

prev_pc      <= PC_RESET;

stalled_inst <= 0;

o_pc         <= 0;

end

else begin

// ── Pipeline register update ──────────────────────────────

// Advance when:

//   (a) stage enabled and not stalled, OR

//   (b) stage enabled, stalled, but output is currently 0

//       (first valid instruction after reset), OR

//   (c) writeback forces a PC change (trap redirect)

if ((ce && !stall_bit)

|| (stall_bit && !o_ce && ce)

|| i_writeback_change_pc) begin

o_iaddr <= iaddr_d;

// restore saved values if returning from stall

o_pc   <= stall_q ? stalled_pc   : prev_pc;

o_inst <= stall_q ? stalled_inst : i_inst;

end

// ── Clock-enable output to Decode ─────────────────────────

if (i_flush && !stall_bit)

o_ce <= 0;           // Section 4.6: flush wins

else if (!stall_bit)

o_ce <= ce_d;        // normal: follow combinational next

else if (stall_bit && !i_stall)

o_ce <= 0;           // fetch stalled but ID is free → bubble

// ── Stall edge detection & snapshot ──────────────────────

stall_q <= i_stall || stall_fetch;

if (stall_bit && !stall_q) begin

stalled_pc   <= prev_pc;   // save before bus moves on

stalled_inst <= i_inst;

end

// ── PC alignment delay ────────────────────────────────────

prev_pc <= o_iaddr;

end

end

// ── Section 4.1 / 4.3 : Combinational PC & CE Logic ─────────────────

always @* begin

iaddr_d     = 0;

ce_d        = 0;

stall_fetch = i_stall;   // propagate downstream stall

if (i_writeback_change_pc) begin      // Priority 1: trap / MRET

iaddr_d = i_writeback_next_pc;

ce_d    = 0;          // bubble

end

else if (i_alu_change_pc) begin        // Priority 2: branch / jump

iaddr_d = i_alu_next_pc;

ce_d    = 0;          // bubble

end

else begin                             // Priority 3: sequential

iaddr_d = o_iaddr + 32'd4;

ce_d    = ce;

end

end

endmodule

# 7.  Key Design Decisions & Observations

## 7.1  Why is ce separate from o_ce?

ce is the internal enable that drives o_stb_inst. o_ce is the pipeline clock-enable forwarded to the Decode stage. They differ because the Decode stage needs to see the bubble one clock edge later than when the fetch stage itself stops requesting. Separating them lets the designer control both edges independently.

## 7.2  Writeback priority over ALU

In the combinational block, i_writeback_change_pc is tested before i_alu_change_pc. This reflects architectural priority: a trap or MRET must always win against a speculative branch result. Both cannot legally be high simultaneously in a correct implementation, but the priority encoding protects against unexpected corner cases during bring-up.

## 7.3  stall_fetch initialized to i_stall

The combinational block sets stall_fetch = i_stall as the base case before any other logic. This ensures stall_fetch is never X (undefined) and that a downstream stall is always reflected even if none of the PC-change branches is taken.

## 7.4  The third update condition in the sequential block

The condition (stall_bit && !o_ce && ce) handles the corner case right after reset: o_ce is 0 (no instruction has yet been delivered to Decode) and ce is 1 (fetch is active). Without this arm, the very first instruction would be lost because stall_bit could be high (memory not yet acked) while o_ce is still 0.

## 7.5  No compressed instruction (RVC) support

The PC is always incremented by exactly 4 (o_iaddr + 32'd4). This design is RV32I-only. Adding RVC (C extension) would require a half-word alignment check and a two-byte increment for 16-bit instructions, plus a 32-bit instruction assembler across fetch boundaries.

# 8.  Testbench Guidance

Since you are writing testbenches for each module, here are the essential test scenarios for rv32i_fetch:

| Test Case | Stimulus | Expected Observable |
| --- | --- | --- |
| Reset behaviour | Assert i_rst_n = 0 for 2 cycles then release | o_iaddr = PC_RESET, o_ce = 0, o_stb_inst = 0 during reset. |
| Sequential fetch | i_ack_inst = 1 every cycle, no stalls | o_iaddr increments by 4 each cycle; o_pc = o_iaddr delayed 1; o_ce = 1. |
| Memory wait-state | Hold i_ack_inst = 0 for 3 cycles then assert | o_iaddr freezes; o_inst / o_pc frozen; o_ce = 0. Advance after ack. |
| Branch taken | Assert i_alu_change_pc with target for 1 cycle | o_ce = 0 (bubble) for 1–2 cycles; o_iaddr jumps to target. |
| Trap (writeback redirect) | Assert i_writeback_change_pc with handler address | Same as branch but handler address used; writeback wins if both asserted. |
| Downstream stall | Assert i_stall for 3 cycles during normal fetch | o_inst and o_pc frozen; on release, same instruction presented. |
| Flush | Assert i_flush for 1 cycle | o_ce = 0 next cycle; instruction discarded. |
| Stall + branch overlap | Assert i_stall while i_alu_change_pc is also high | Branch target loaded but no bubble until stall releases. |

Tip: Use a simple memory model in the testbench that returns 32'h0000_0013 (the NOP instruction, ADDI x0, x0, 0) as the default instruction. This gives you a legal, executable word without needing a real instruction ROM during unit testing.

# 9.  Glossary

| Term | Definition |
| --- | --- |
| Pipeline Bubble | A cycle in which no valid instruction is presented to the Decode stage (o_ce = 0). Used to flush wrongly-fetched instructions after a PC redirect. |
| PC Redirect | Any event that changes the Program Counter to a non-sequential address. Sources: taken branch (ALU), JAL/JALR (ALU), trap entry, MRET (Writeback). |
| Stall | A condition in which one or more pipeline stages freeze their outputs for one or more cycles. Can be caused by memory latency, load-use hazards, or structural hazards. |
| Flush | Explicit invalidation of the instruction currently in the fetch stage, forcing o_ce = 0 without necessarily changing the PC. |
| Clock Enable (CE) | A signal that gates the update of pipeline registers. When CE = 0, the register holds its previous value (equivalent to inserting a NOP from the next stage's perspective). |
| Strobe (STB) | In Wishbone terminology, a signal from master to slave indicating a valid transaction is being requested this cycle. |
| Acknowledge (ACK) | In Wishbone terminology, a signal from slave to master indicating the transaction has completed and data is valid. |
| stall_q | One-cycle delayed version of the stall condition used to detect the first cycle of a new stall (rising edge detection). |
| prev_pc | A one-cycle delayed copy of o_iaddr that aligns the PC value with the instruction data coming back from memory. |
| RV32I | The base integer instruction set for 32-bit RISC-V processors. Includes 47 instructions across R, I, S, B, U, and J encoding formats. |